Course: ENSF 460 – Embedded Software and Hardware Systems
Assignment #: Assignment 4
Instructor Name: Benjamin Tan
Student Name(s): Stephen Ravelo, Aaron Lauang, Alexa González
Lab Section: B02
Date submitted: November 7, 2025

# Peripheral Initialization and Setup

Peripherals and setup are done at the beginning of function main.

## main.c

int main(void) {

// Run once code

AD1PCFG = 0xFFFF; /* keep this line as it sets I/O pins that can also be analog to be digital */

newClk(500);

timerInit();

IOinit();

IPC4bits.CNIP = 6;

IFS1bits.CNIF = 0;

IEC1bits.CNIE = 1;

/* Let's set up our UART */

InitUART2();

_state = STATE_MODE_0;

CN_event = 0;

uint16_t ADC1_val = do_ADC();

uint16_t ADC1_last = ADC1_val + 16;

…

The ADC input is configured in the ADC.c file. For this assignment we are using AN12(Pin 15) as the analog input pin:

## ADC.c

// Select and configure ADC input

AD1CHSbits.CH0NA = 0;

AD1CHSbits.CH0SA = 0b1100;

AD1PCFGbits.PCFG12 = 0;

AD1CSSLbits.CSSL12 = 0;

The do_ADC function in the ADC.c file configures, starts, and performs a single analog-to-digital conversion on channel AN5. It sets up all necessary ADC registers, starts sampling, waits until the conversion is complete, reads the digital result from ADC1BUF0, and then turns off the ADC module. Finally, it returns the converted 10-bit digital value.

## ADC.c

uint16_t do_ADC(void) {

uint16_t ADCvalue; // 16 bit register used to hold ADC converted digital output ADC1BUF0

/* ------------- ADC INITIALIZATION ------------------*/

// Configure ADC by setting bits in AD1CON1 register

AD1CON1bits.ADSIDL = 0;

AD1CON1bits.FORM = 0b00;

AD1CON1bits.SSRC = 0b111;

AD1CON1bits.ASAM = 0;

AD1CON2bits.VCFG = 0b000; // Selects AVDD, AVSS (supply voltage to PIC) as Vref

// Configure ADC by setting bits in AD1CON2

AD1CON2bits.CSCNA = 0;

//    AD1CON2bits.SMPI = 0b000;

AD1CON2bits.BUFM = 0;

AD1CON2bits.ALTS = 0;

AD1CON3bits.ADRC = 0; // Use system clock

//Configure the ADC?s sample time by setting bits in AD1CON3

// Ensure sample time is 1/10th of signal being sampled

AD1CON3bits.SAMC = 0b11111;

// Select and configure ADC input

AD1CHSbits.CH0NA = 0;

AD1CHSbits.CH0SA = 0b1100;

AD1PCFGbits.PCFG12 = 0;

AD1CSSLbits.CSSL12 = 0;

/* ------------- ADC SAMPLING AND CONVERSION ------------------*/

AD1CON1bits.ADON = 1; // turn on ADC module

AD1CON1bits.SAMP=1; //Start Sampling, Conversion starts automatically after SSRC and SAMC settings

while(AD1CON1bits.DONE==0) {}

ADCvalue = ADC1BUF0; // ADC output is stored in ADC1BUF0 as this point

AD1CON1bits.SAMP=0; //Stop sampling

AD1CON1bits.ADON=0; //Turn off ADC, ADC value stored in ADC1BUF0;

return (ADCvalue); //returns 10 bit ADC output stored in ADC1BIF0 to calling function

}

This function reads and sends ADC values for about 10 seconds. It sends the message "START_READING" before sampling and "STOP_READING" after finishing, which can be used by the Python program to detect when data collection starts and ends. The sampling rate depends on the samples parameter, which must be less than 100.

## ADC.c

void read_ADC(uint16_t samples) {

uint16_t delay_time = 10000/samples - samples;

Disp2String("START_READING\n");

for (int i = 0; i < samples; i++) {

Disp2Dec(do_ADC());

delay_ms(delay_time);

}

Disp2String("STOP_READING\n");

}

timerInit() is found in defined in timer.c. The function configures timer 2 to our desired parameters.

## timer.c

void timerInit() {

// TMR2 config

T2CONbits.T32 = 0;

T2CONbits.TCKPS = 3;    // set prescalar to 256

T2CONbits.TCS = 0;

T2CONbits.TGATE = 0;

T2CONbits.TSIDL = 0;

IPC1bits.T2IP = 2;

IFS0bits.T2IF = 0;

IEC0bits.T2IE = 1;

}

void delay_ms(uint16_t time_ms) {

PR2 = 1 * time_ms;      // PR2 coefficient: 1 ~= 0.001 * 250000 / 256

TMR2 = 0;

T2CONbits.TON = 1;

// Idle until timer 2 interrupt or valid input detected

while (T2CONbits.TON == 1) {

Idle();

if (IOcheck()) { break; }

}

return;

}

Using a pre-scaler of 256 and clock frequency 500 kHz, our PR2 calculation comes in to about 2.3% off 1. Using this value allows us to be accurate for most delays and utilize create delays up to the capacity of an unsigned 16-bit integer (65535 ms).

We then call IOinit() from IOs.c. The function configures our pin I/O.

## IOs.c

state_t _state;

// Initialize peripheral IO

void IOinit() {

// PB1

TRISBbits.TRISB7 = 1;

CNPU2bits.CN23PUE = 1;

CNEN2bits.CN23IE = 1;

// PB2

TRISBbits.TRISB4 = 1;

CNPU1bits.CN1PUE = 1;

CNEN1bits.CN1IE = 1;

}

Interrupts are enabled for each of the buttons.

# Program Logic

Our logic occurs in the main while(1) loop. The while(1) keeps the program running continuously. Inside it, in STATE_MODE_0, it enters a sub-loop that reads the ADC and repeatedly displays the value via UART (with a 1-second delay). Switching between states can be done using PB1. In STATE_MODE_1, the program prompts the user to press PB2 to start sampling data over 10 seconds. This communicates with our python program ADC.py which will create a graph of the buffer and voltage values sent by the ADC.

## main.c

…

// Main loop

while(1) {

switch(_state) {

/*

* Displays number of '*' proportional to displayed ADC value

* Pressing PB1 in this state triggers a transition to STATE_MODE_1

*/

case STATE_MODE_0:

while(_state == STATE_MODE_0) {

ADC1_val = do_ADC();

if (ADC1_val != ADC1_last) {

DispMode0(ADC1_val);

}

ADC1_last = ADC1_val;

delay_ms(1000);

if (CN_event) {

uint16_t IO_event = IOcheck();

if (IO_event == 1) {

_state = STATE_MODE_1;

}

CN_event = 0;

}

}

break;

/*

* Pressing PB1 in this state triggers a transition to STATE_MODE_2

* Pressing PB2 in this state samples ADC data for 10 seconds

* A 'hidden' state STATE_READ_ADC protects data sampling

*/

case STATE_MODE_1:

while (_state == STATE_MODE_1) {

Disp2String("\033[2J\033[1;1HMode 1: Press PB2 to start data streaming\n");

delay_ms(50);

Idle();

delay_ms(50);

if (CN_event) {

uint16_t IO_event = IOcheck();

if (IO_event == 1) {

_state = STATE_MODE_0;

DispMode0(ADC1_val);

} else if (IO_event == 2) {

_state = STATE_READ_ADC;

read_ADC(99);

_state = STATE_MODE_1;

Idle();

}

CN_event = 0;

}

}

break;

default:

Idle();

break;

}

}

…

## ADC.py

'''

File Name: ADC.py

Assignment: Assignment 4

Lab Section: B02

Completed by: Stephen Ravelo, Aaron Lauang, Alexa Gonzalez

Submission Date: November 7, 2025

'''

import serial

import time

import pandas as pd

import matplotlib.pyplot as plt

# port_name = '/dev/tty.usbserial-0001'

port_name = 'COM4'

baud_rate = 4800

buffer = []

voltage = []

sample_times = []

try:

# Connect python program to USB UART

ser = serial.Serial(port_name, baud_rate)

print(f"Connected to {port_name}.")

time.sleep(2)

while True:

if ser.in_waiting > 0:

msg = ser.readline().decode().replace('\x00', '')

if msg == "START_READING\n":

print("Sampling data...")

t0 = time.time()

# Receive ADC values from the C program that is transmitted to the UART

while 1:

if ser.in_waiting > 0:

sample_msg = ser.readline().decode().replace('\x00', '').replace('\n', '')

if (sample_msg == "STOP_READING"):

break

buffer_data = sample_msg.lstrip('0')

if buffer_data == '':

buffer.append(0)

voltage.append(0)

else:

buffer.append(int(buffer_data))

voltage.append(float(buffer_data) * 0.00322265625)

sample_times.append(time.time() - t0)

# Print buffer, voltage, and time values into the terminal

print("Digital ADC Buffer Values:")

print(buffer)

print("ADC Input Voltage Values:")

print(voltage)

print("Sample Times:")

print(sample_times)

# Save data into a CSV file

df = pd.DataFrame({

'Sample Time (seconds)': sample_times,

'Digital ADC Buffer': buffer,

'ADC Input Voltage (volts)': voltage

})

df.to_csv('data.csv', index=False)

# Output the graphs of buffer vs. time, and voltage vs. time

fig, (ax1, ax2) = plt.subplots(2, 1, figsize=(8, 6), sharex=True)

ax1.plot(sample_times, buffer, 'b-o', label='Buffer')

ax1.set_ylabel('Digital ADC Buffer')

ax1.set_xlabel('Sample Time (seconds)')

ax1.set_title('Digital ADC Buffer vs. Sample Time')

ax1.legend()

ax1.grid(True)

ax2.plot(sample_times, voltage, 'r-o', label='Voltage')

ax2.set_ylabel('ADC Input Voltage (volts)')

ax2.set_xlabel('Sample Time (seconds)')

ax2.set_title('ADC Input Voltage vs. Sample Time')

ax2.legend()

ax2.grid(True)

plt.tight_layout()

plt.show()

else:

print(msg)

# Clear data

buffer.clear()

voltage.clear()

sample_times.clear()

continue

time.sleep(0.1)

except serial.SerialException as e:

print(f"Error opening or communicating with serial port: {e}.")

except KeyboardInterrupt:

print("Exiting.")

finally:

ser.close()

print("Serial port closed.")

# State Machine Design

While in STATE_MODE_0, the program samples the ADC every second and displays several star symbols proportional to the buffer obtained. Pressing PB1 results in a state switch to STATE_MODE_1.

While in STATE_MODE_, the program remains in Idle() until either PB is pressed. Pressing PB1 results in a state switch to STATE_MODE_0. Pressing PB2 results in a state switch to STATE_READ_ADC.

While in STATE_READ_ADC, the program sends a START_READING message to the UART and while sample the ADC a given number of times over 10 seconds. Afterwards, it sends a STOP_READING message and returns to STATE_MODE_1.